- methodology

ALGORITHM

Image Preprocessing

The images in the given dataset are in a lab setup. Existing overhead lighting There are elements present in all the images that do not pertain to the snake body and patches of extreme brightness and dull spots. Hence removing unwanted elements and normalisation is essential. For the purpose of this paper, we have adopted two methods:

  - Manual Selection of Edge Points: The image is first converted to grayscale. Truncated thresholding is applied to eliminate extreme values beyond a certain threshold, outperforming other thresholding methods by preserving essential data features while reducing the impact of outliers or noise in the dataset. Image sharpening is performed using adaptive histogram equalisation that results in a far more enhanced and balanced image. We enable the user to manually click on the edge points encompassing the snake body. On applying the warped perspective transform, further image processing techniques described in Section B are followed.

  - Image Segmentation: To perform segmentation, the approach is to subtract the background and only retain the foreground. We first convert the image to a grayscale image for processing. We detect contours in the binary image to compress horizontal, diagonal and vertical segments, keeping only the end points. The largest contour is selected and is drawn on an empty mask of zeros. This creates a black background for the object. Using bitwise and the initial image and the mask are combined resulting in a well segmented image showing only the snake’s body with scales against a black background. Following this truncated thresholding and image sharpening is performed, as in the manual selection method explained in Methodology Section A(i).

Canny Edge Detection

Canny Edge Detection, a standard edge detection algorithm has been used to detect edges. However, instead of utilising OpenCV’s inbuilt canny function, we have designed our own Canny edge detector which seems to outperform the inbuilt function for our cause. The difference occurs due to the custom thresholding where we are at liberty to test out different upper and lower threshold values according to the purpose. The inbuilt function relies on fixed, automatic thresholding which is not optimal for all images. With custom implementation, the noise removal and blurring process is in our hands, with the kernel size being determined by us for our application. Even with the Hysteresis Thresholding step, the custom implementation allows us to fine-tune parameters to our project’s needs. As the images we have are quite noisy with unnecessary information and caried lighting, this is an exceedingly important step.

Morphological Operations

Morphological operations are important for enhancing the quality of edges in the image and for improving the accuracy of contour detection. These operations are crucial for preparing the image after canny edge detection for contour detection and counting.

- Morphological Dilation: The boundaries of the foreground object are expanded by adding pixels around it, making It larger. It closes any small gaps and holes within the object, making it more solid and making the features thicker. In our case, we see that dilation sometimes causes overlap of features due to thickening of lines.

- Morphological Erosion: Any noise and unwanted details in the image by shrinking the boundaries of the foreground object, making it smaller. It is useful in our case, to distinguish boundaries of objects in the image, as it erodes away the outer pixels, leaving boundary pixels. The thinning effect it has proves particularly useful in the contour detection step.

Contour Finding and Counting

For this step, we first take the complement of the morphed and thresholded image. The contours of this inverted image are found and their edges retained. On iterating through the list of contours detected, the contour area of each contour is found. Using this, the average area is calculated. If the area of the ith contour is greater than the average area * scale factor ‘k1’ and less than the average area * scale factor ‘k2’, the scale counter is incremented. This allows for slight deviations and room for error in contour area upto factors k1 and k2.

Finally, the contours found are drawn onto the original image without the mask and the contour count is displayed.

TABLE I
 DIFFERENT STAGES OF METHOD 1 FOR RAT SNAKE: MANUAL SELECTION OF EDGE POINTS

| Original Colour | Gray | Truncated Thresholding |
| --- | --- | --- |
| Image Sharpening | Cropped and Warped Image | Canny Edge Detection |
| Morphological Operations | Thresholding | Output Image |

TABLE II

DIFFERENT STAGES OF METHOD 1 FOR TRINKET SNAKE: MANUAL SELECTION OF EDGE POINTS

| Original Colour | Gray | Truncated Thresholding |
| --- | --- | --- |
| Image Sharpening | Cropped and Warped Image | Canny Edge Detection |
| Morphological Operations | Thresholding | Output Image |

Parameters involved in method 1:

A. Threshold values for Canny

Fixed parameters:

  - Kernel for Dilation: (9,9)

  - Kernel for Erosion: (9,9)

  - Contour Area: area>avg_area*0.3 and area<avg_area*8

TABLE I (a)

| Lower Threshold | Upper Threshold | Canny Image | Final Image | Type of Snake |
| --- | --- | --- | --- | --- |
| 20 | 255 |  | Scale Count = 88(Most are not scales) | Rat |
| 74 | 255 |  | Scale Count = 5(Scales are not detected) | Rat |
| 34 | 205 |  | Scale Count = 50 | Rat |
| 34 | 170 |  | Scale Count = 57 - BEST | Rat |
| 80 | 255 |  | Scale Count = 27(Scales not detected) | Trinket |
| 40 | 255 |  | Scale Count = 156 | Trinket |
| 20 | 205 |  | Scale Count = 310(Most are not scales) | Trinket |
| 40 | 197 |  | Scale Count = 152 - BEST | Trinket |

As is visible in the TABLE 1(a), different values of upper and lower threshold affect the scale count drastically. In the first case, although the count shows 88 (highest count), the image is overpopulated with minuscule contours that do not account for actual scales. This is because the lower threshold is so low that it detects the dark ridges between scales as contours. Then the value dips very suddenly to 5 as the lower threshold is so high that even slightly darkened pixels that are scales, are not coming forth. The count rises after this point when both upper and lower thresholds are decreased slightly showing that it is reaching an optimum value which is obtained at (34, 70).

B. Kernel for Dilation and Erosion

Fixed parameters

a) Threshold values for Canny = 40,197

b) Contour Area: area>avg_area*0.2 and area<avg_area*4

| Kernel for Dilation | Kernel for Erosion | Morphological Image | Final Image | Type of Snake |
| --- | --- | --- | --- | --- |
| 5,5 | 5,5 |  | Scale Count = 70(overlapping scales) | Rat |
| 7,7 | 7,7 |  | Scale Count = 61(extra scales detected) | Rat |
| 9,9 | 9,9 |  | Scale Count = 57 -BEST | Rat |
| 5,5 | 5,5 |  | Scale Count = 152 - BEST | Trinket |
| 7,7 | 7,7 |  | Scale Count = 121(Some scales not detected) | Trinket |
| 9,9 | 9,9 |  | Scale Count = 101 (Scales not detected) | Trinket |

For the rat snake using method 1, we see that the kernel (9,9) performs the best. The rat snake scales are larger and well-defined as compared to those of a trinket snake, therefore a larger kernel provides a higher accuracy. Beyond a (9,9) kernel, the performance starts degrading again. On the other hand, a trinket snake is a snake with much smaller and thinner scales. Therefore our algorithm reaches its optimum value for the trinket snake at a (5,5) kernel.

C. Contour Area

Fixed parameters

a) Threshold values for Canny = 34,170

b) Kernel for Dilation and Kernel for Erosion = (9,9)

| Lower value | Higher Value | Final Image | Snake Type |
| --- | --- | --- | --- |
| area>avg_area*0.5 | area<avg_area*8 | Scale Count = 44 | Rat |
| area>avg_area*0.3 | area<avg_area*2 | Scale Count = 48 | Rat |
| area>avg_area*0.3 | area<avg_area*4 | Scale Count = 51 | Rat |
| area>avg_area*0.3 | area<avg_area*8 | Scale Count = 57 - BEST | Rat |
| area>avg_area*0.5 | area<avg_area*8 | Scale Count = 104 | Trinket |
| area>avg_area*0.2 | area<avg_area*2 | Scale Count = 137 | Trinket |
| area>avg_area*0.2 | area<avg_area*4 | Scale Count = 152 - BEST | Trinket |
| area>avg_area*0.1 | area<avg_area*9 | Scale Count = 57 | Trinket |

It is seen that the best performance for trinket snake is given by area > avg_area*0.2 and area < avg_area*4. When the upper margin for area is too large, for instance with area< avg_area*8 and area< avg_area*9, parts of the snake that are not contours are being detected as the margin for error is too large. It is unrealistic to accept values 8 and 9 times the average in this case. However when the upper threshold is twice the average and lower threshold is 0.2 times the average it seems to underperform owing to a lack of accurate contours to include. The space for error is practically, given that the images all differ in lighting, brightness, intensity and scale quality. Therefore, a median value was tried which yielded the best results, giving 152 contours which is closest to the actual values. However this is only because the scales on a trinket snake are smaller than those of a snake with larger scales, for instance, the rat snake. In the case of the rat snake the opposite occurs. As the scales are larger and more defined, it is advantageous to keep the higher threshold 8 or 9 times the average as one is more likely to include a scale in them.

TABLE III
 DIFFERENT STAGES OF METHOD 2 FOR RAT SNAKE: IMAGE SEGMENTATION

| Original Colour | Gray | Inverse Binary Thresholding |
| --- | --- | --- |
| Segmented Image | Image Sharpening | Canny Edge Detection |
| Morphological Operations | Binary Thresholding | Output Image |

A. Threshold values for Canny

Fixed parameters

a) Kernel for Dilation: (9,9)

b) Kernel for Erosion: (9,9)

c) Contour Area: area>avg_area*0.09 and area<avg_area*1.5

| Lower Threshold | Upper Threshold | Canny Image | Final Image |
| --- | --- | --- | --- |
| 20 | 255 |  | Scale Count = 65(Not scales) |
| 50 | 255 |  | Scale Count = 48 |
| 80 | 215 |  | Scale Count = 10(Scales not detected) |
| 40 | 195 |  | Scale Count = 53 - BEST |

B. Kernel for Dilation and Erosion

Fixed parameters

- Threshold values for Canny = 45,195

- Contour Area: area>avg_area*0.09 and area<avg_area*1.5

| Kernel for Dilation | Kernel for Erosion | Morphological Image | Final mage |
| --- | --- | --- | --- |
| 5,5 | 5,5 |  | Scale Count = 83 |
| 7,7 | 7,7 |  | Scale Count = 61 - BEST |
| 9,9 | 9,9 |  | Scale Count = 53 |

C. Contour Area

Fixed parameters

a) Threshold values for Canny = 45,195

b) Kernel for Dilation and Kernel for Erosion = (9,9)

| Lower value | Higher Value | Final Image |
| --- | --- | --- |
| area>avg_area*0.4 | area<avg_area*6 | Scale Count = 24 |
| area>avg_area*0.2 | area<avg_area*4 | Scale Count = 42 - BEST |
| area>avg_area*0.2 | area<avg_area*2 | Scale Count = 25 |
| area>avg_area*0.09 | area<avg_area*1.5 | Scale Count = 25 |

- results and discussion

Our final observations and inferences from our prescribed methodology comprise two main components: the scale count and the independent parameters.

- For a rat snake using methodology 1, we see the result below:

The actual number of scales is 62 when counted by hand. Using Method 1 we have received a count of 57 which is well within the 85% accuracy margin. Although the contours do not exactly overlap with all scales, the accuracy is high, showing that necessary features have been included and the parameters have been estimated successfully. There is one region in the final output where there are two contours being detected in one snake scale. This can be fixed by either creating a deep learning model to learn parameters or by implementing a penalty system for misclassification or overclassification.

The parameters of the model are:

Threshold: Lower threshold = 34, Upper Threshold = 170

Kernel Size: 9,9

Area Criteria: area>avg_area*0.3 and area<avg_area*8

Accuracy: 91%

- For a rat snake using methodology 2, we see the result below:

The observations for image segmentation are very similar to those of manual selection edge points.

The actual number of scales is 62 when counted by hand. Using Method 2 we have received a count of 56 which is well within the 85% accuracy margin. Although the contours do not exactly overlap with all scales, the accuracy is quite high, showing that necessary features have been included and the parameters have been estimated successfully. There is one region in the final output where there are two contours being detected in one snake scale. This can be fixed by either creating a deep learning model to learn parameters or by implementing a penalty system for misclassification or overclassification.  The lower part of the image does not show scales. This is owing to the lighting in the laboratory being directly over this part of thr observation glass.

The parameters of the model are:

Threshold: Lower threshold = 40, Upper Threshold = 195

Kernel Size: 7,7

Area Criteria: area>avg_area*0.2 and area<avg_area*1.5

Accuracy: 90.32%

- Conclusions

The dataset comprises snakes of various species, observed in a laboratory setup. The images all differ in size, lighting and quality. Some snakes have scratches on their scales, making it difficult to accurately count scales. The images that have been used have gone through various steps of preprocessing, such as sharpening, thresholding, segmentation/ cropping and then have been further  been passed through a canny edge detector, morphological dilation and erosion, to detect count and draw contours onto the original images with an accuracy of 85%..

Futurework

- Acknowledgement

We would like to take this opportunity to thank our professors – Professor Siddalingaswamy P C, Dr. Muralikrishna SN and dedicated Phd scholar Mayur Sir for their invaluable guidance and mentorship throughout the duration of the project.Their endless support and expertise about the subject helped us in our growth in the subject and the project’s success.

- References